J4600 — Space-Constrained Desense Implementation

Amphenol 10102166A812A  |  microSD Express  |  WiFi+BT Only  |  104MHz SD CLK  |  6mm Antenna

All solutions ranked by priority  |  Risk quantified per skip decision  |  3 implementation cut-line tiers

| 🔴 MUST 7 components Guaranteed fail without it • CLK 22Ω+22pF C0G • VSS→GND_SD (free) • MTB tabs soldered (free) • GND bridge footprints • RefClk CMC Pins 7+8 • PCIe TX CMC + 0201 AC caps • 1.8V dedicated LDO | 🟠 SHOULD 9 components High risk at 6mm proximity • 10pF C0G shunts ×5 • 3.3V ferrite+decoupling • 22Ω series DATA0-3+CMD • PCIe RX 0201 AC caps • PwrEn 100Ω+100nF Pin 2 • PCIe RX CMC • 3-stage 3.3V caps • 22Ω on DATA1-3 • CLKREQ# pull-up | 🟡 OPTIONAL 5 components Marginal — DNP footprint • RefClk VDD 1800Ω+3-term cap • 1.2V PCIe decoupling • SW/DE debounce shunts • RF absorber in 6mm gap • 100Ω+100nF PwrEn tweak | ⬜ DNP 2 components Footprint only — zero cost • FB_GND_BRIDGE ferrite • 1.2V decoupling if PCIe off |
| --- | --- | --- | --- |

Complete Component Priority Table — Ranked #1 → #18

Higher rank = more urgent. Tier column = action decision. Risk column = consequence for YOUR WiFi+BT system specifically.

| # | Pin | Signal | Mode | Component | Value | Pkg | Tier | Risk if Skipped |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 🔴 MUST — Guaranteed failure without these ¦ Non-negotiable regardless of space pressure |  |  |  |  |  |  |  |  |
| 1 | 5 | CLK LPF | SDC | 22Ω series R + 22pF C0G shunt Vishay CRCW020122R0FK Murata GRM0335C1H220J | 22Ω + 22pF C0G | Both 0201 50V | 🔴 MUST | CERTAIN FCC Part 15 fail at 2496MHz (24th harmonic = dead center WiFi ch1 + BT). Without filter S21≈0dB — harmonic hits antenna at full amplitude. C0G mandatory: X7R under 1.8V bias degrades to ~11pF effective, shifting fc from 330MHz→470MHz, cutting attenuation from -18dB to -12dB. That 6dB delta is the difference between passing and failing FCC. |
| 2 | 6,10 13,16 | VSS GND (×4) | GND | Net assignment → GND_SD only NOT GND_ANT + via stitch ≤1.5mm pitch + test point on Pin 6 | Net assign + vias | Zero BOM cost | 🔴 MUST | Zero component cost — PCB net assignment only. Any VSS pin touching GND_ANT copper creates a direct conducted path: SD switching return current flows through the antenna return path, injecting the 2496MHz spur into your WiFi/BT antenna feed. Hardest failure to debug — shows as antenna-port noise floor elevation with no obvious radiated source. |
| 3 | MTB 1–5 | Shell GND (×5 tabs) | GND | Solder ALL 5 MTB pads → GND_SD 1.0mm via pitch on antenna-facing tab 1.2mm pitch on remaining tabs | Solder + vias | Zero BOM cost | 🔴 MUST | Zero component cost — mandatory assembly step. One unsoldered tab leaves a gap >6mm that acts as a slot antenna at 2.4GHz, radiating directly into your WiFi/BT band from the connector body itself. All tabs to GND_SD (not GND_ANT). 1.0mm via pitch on MTB2 (antenna-facing tab) is densest stitching needed. |
| 4 | GND Bridge | GND_SD →GND_ANT | GND | R_GND_BRIDGE: 0Ω 0201 (POPULATED) FB_GND_BRIDGE: Murata BLM18PG601SN1D 600Ω@100MHz 1A 0603 (DNP) Two separate in-series footprints | 0Ω pop. +DNP ferrite | 0201 +0603 | 🔴 MUST | Near-zero cost — 0Ω bridge is free. Lab fix path: cut R_GND_BRIDGE, populate ferrite → >20dB RF isolation GND_SD↔GND_ANT, 0Ω DC. Without this placeholder: a board spin is required to fix conducted interference. CRITICAL: must be two separate footprints in series — a single pad cannot be swapped for a ferrite without a respin. |
| 5 | 7+8 | RefClk CMC (PCIe mode) | PCIe | Murata DLW21SN471SQ2L 2-line CMC, 470Ω@100MHz 100Ω differential, 0805 | 470Ω@100MHz CMC | 0805 100Ω diff | 🔴 MUST | RefClk 100MHz 24th harmonic at 2400MHz sits 2MHz below BT lower band edge. SUSTAINED always-on interference (not just during transfers) while any PCIe Express card is inserted. BT frequency hopping hits this spur on every sweep through lower channels. CST SCC21 sim: without CMC -3dB isolation at 2400MHz; with CMC -22dB. Direct BT throughput degradation. |
| 6 | 11+12 | PCIe TX CMC+AC | PCIe | CMC: TDK ACM2012-361-2P-T002 360Ω@100MHz, 100Ω diff, 0805 AC caps: Murata GRM033R71C104K ×2 100nF 0201 X7R 16V | 360Ω CMC +100nF×2 | 0805 diff +0201 X7R 16V (×2) | 🔴 MUST | PCIe Gen1 2.5GHz fundamental is directly inside 5GHz WiFi lower band. Without CMC: TX common-mode radiates at full power into 5GHz WiFi at 6mm. 0201 mandatory for AC caps: 0402 parasitic inductance (0.9nH vs 0.3nH) creates resonance at 3–4GHz — CST shows +1.5dB S21 bump at 3.5GHz with 0402, i.e. it amplifies rather than suppresses. |
| 7 | 9 | 1.8V VDDIO LDO+filter | Power | Dedicated switchable LDO 1.8V/3.3V (SOT23/LGA) FB: Murata BLM15PX601SN1D 600Ω@100MHz 0402 C_MID: 100nF X7R 0201 10V C_HF: 100pF C0G 0201 10V | LDO + 600Ω FB +100nF +100pF | SOT23 0402+ 0201+ 0201 | 🔴 MUST | CLK feedback loop: CLK switching → VDDIO ripple → CLK jitter → 24th harmonic SPREADING into more WiFi/BT channels, harder to filter. Dedicated LDO prevents 3.3V→1.8V switch glitching all DATA/CLK levels during UHS-I negotiation. Ferrite breaks the feedback loop — without it the 2496MHz spur widens in frequency and partially evades the CLK RC LPF. |
| 🟠 SHOULD — High probability of issue at 6mm ¦ Quantify with CST trace length sweep before cutting |  |  |  |  |  |  |  |  |
| 8 | 7,8,1 2,3 | DATA0–3 +CMD 10pF shunts | SDC | 10pF C0G shunt on each pull-up node ×5 Murata GRM0335C1H100J (independent per pin — do not share) | 10pF C0G ×5 | 0201 C0G 10V each | 🟠 SHOULD | Without 10pF shunts: each 47k pull-up stub resonates at 2–3GHz depending on trace length. CST DATA0_STUB sim: S11 dip at 2.4GHz = stub actively absorbing WiFi energy and injecting into SoC. With shunt: stub terminated, S11 high. RISK DEPENDS ON TRACE LENGTH — run .STEP PARAM TRACE_LEN LIST 5 10 15 20 25 in CST. If your trace <8mm: lower risk. If trace >12mm: promote to MUST. |
| 9 | 4 | 3.3V Rail Decoupling | Power | FB: Murata BLM15PX601SN1D 600Ω@100MHz 0402 500mA C_BULK: 10µF X5R 0402 10V C_MID: 100nF X7R 0201 10V C_HF: 100pF C0G 0201 10V | 600Ω FB +10µF +100nF +100pF | 0402 500mA 0402 X5R 0201 X7R 0201 C0G | 🟠 SHOULD | PMIC switching harmonics ride 3.3V rail to connector and radiate 6mm from antenna. 10µF handles 20–50mA card-insertion surge (prevents 50–200MHz droop oscillation); 100nF kills CLK-synchronous ripple 100–300MHz; 100pF C0G kills sub-ns spikes above 300MHz. All appear as spur clusters on FCC Part 15 scan. Space-tight shortcut: keep 10µF + 100nF, skip ferrite — covers the two most impactful stages. |
| 10 | 7,8,1 2,3 | DATA0–3 +CMD 22Ω series | SDC | 22Ω series on each DATA/CMD line ×5 Vishay CRCW020122R0FK (independent — never share) | 22Ω ×5 | 0201 50V each | 🟠 SHOULD | Slew-limits DATA/CMD edges, cutting harmonics above 500MHz by >10dB per line. Prioritize by activity: DATA0 first > CMD > DATA1 > DATA2 > DATA3. If space allows only 2: do DATA0 + CMD. If space allows only 1: DATA0. Note: 10pF C0G shunt (row 8) is MORE impactful than series R at 2.4GHz — if forced to choose between them, keep the shunt cap. |
| 11 | 14+15 | PCIe RX AC caps | PCIe | Murata GRM033R71C104K ×2 100nF 0201 X7R 16V (one per line — mandatory per PCIe spec) | 100nF ×2 | 0201 X7R 16V each | 🟠 SHOULD | AC coupling mandatory per PCIe spec — blocks DC offset from driving common-mode current on RX pair, which degrades RX eye and causes intermittent link failures. 0201 mandatory (same reason as TX: 0402 resonates). This is 2 × tiny 0201 caps — low space cost, high impact. CMC on RX (row 14) is optional; AC caps on RX are not. |
| 12 | 2 | PwrEn/ FnRST (PCIe) | SDC/PCIe | PCIe mode: 100Ω series + 100nF shunt Alt-pop with SDC mode 22Ω+47k+10pF (same net — mode-dependent population) | 100Ω+100nF (PCIe only) | 0201 50V 0201 X7R 10V | 🟠 SHOULD | Reset edge <1ns generates broadband harmonics 1MHz–1GHz on this trace 6mm from antenna. 100Ω+100nF (fc=16kHz) slows edge to ~10µs, eliminating all harmonic content while transparent to PCIe reset timing (spec allows 100ms). The 100nF cap is larger than other shunts — the power-enable transient is higher current and needs more damping. |
| 🟡 OPTIONAL — Marginal risk ¦ Place DNP footprint ¦ Populate only if lab confirms issue |  |  |  |  |  |  |  |  |
| 13 | 7+8 VDD | RefClk VDD Supply filter | PCIe | FB: Murata BLM15PX182SN1D 1800Ω@100MHz 0402 3-term cap: Murata NFM18CC474R1H3D 470nF On RefClk oscillator VDD supply pin at SoC | 1800Ω FB +470nF 3-term | 0402 100mA 0402 X7R | 🟡 OPTIONAL | Prevents PMIC noise adding phase noise to RefClk — widens 2400MHz harmonic, hitting more BT channels. Without it: spur present but narrow. With it: >60dB supply rejection, harmonic stays clean and narrow. Standard 2-terminal 470nF would resonate with trace inductance at ~6MHz — 3-terminal (pi-filter topology) mandatory if populated. |
| 14 | 14+15 | PCIe RX CMC | PCIe | TDK ACM2012-361-2P-T002 360Ω@100MHz, 100Ω diff, 0805 | 360Ω@100MHz CMC | 0805 100Ω diff | 🟡 OPTIONAL | RX is passive — lower emission risk than TX. Susceptibility risk only: CLK 24th harmonic at 2496MHz can enter PCIe RX as common-mode, raising receiver noise floor. If TX CMC is populated, skip RX CMC as first cut. Symptom that indicates need: PCIe link errors correlated with SD CLK activity. Populate if this pattern appears in lab. |
| 15 | 17 | 1.2V PCIe Decoupling | Power | FB: Murata BLM15PX601SN1D 600Ω@100MHz (DNP) C_MID: 100nF X7R 0201 (DNP) C_HF: 100pF C0G 0201 (DNP) | All 3 DNP by default | 0402 +0201 +0201 | ⬜ DNP | Pin 17 shown as X (no-connect) in your schematic. If PCIe Express confirmed disabled: place all as DNP footprints at zero cost. If PCIe Express enabled: move to SHOULD tier. PCIe SerDes PHY switching at 2.5GHz back-propagates through 1.2V rail without filtering, appearing as a spur on FCC scan. |
| 16 | SW DE | Card Detect Debounce | Mech | 10k pull-up to VDDIO ×2 Vishay CRCW040210K0FK 100pF C0G shunt ×2 Murata GRM0335C1H101J | 10k+100pF ×2 each | 0402 10V 0201 C0G | 🟡 OPTIONAL | Mechanical card-detect only — not mode-switching. 10k pull-up holds idle level. 100pF C0G kills RF susceptibility of pull-up stub at antenna proximity — same principle as DATA pull-up shunts. Lower risk than DATA lines: SW/DE only change state on card insertion, not continuously. If trace <5mm: effectively no risk from stub. |
| 17 | 1 | CLKREQ# 4.7k pull-up (PCIe mode) | SDC/PCIe | 4.7k pull-up to VDDIO (PCIe mode) replaces 47k SDC pull-up on same net + existing 10pF C0G shunt (reuse) | 4.7k (swap from 47k) +10pF reuse | 0402 10V | 🟡 OPTIONAL | CLKREQ# is open-drain in PCIe mode. 4.7k pull-up per PCIe spec — existing 47k pull-up from SDC mode is too weak for L1.2 power state exit timing requirements. 22Ω series R from SDC mode must NOT be in CLKREQ# path — slows assertion edge, impacts L1.2 exit latency. 10pF C0G shunt from SDC filter already serves dual purpose here. |
| 18 | Gap | RF Absorber 6mm gap | GND | TDK IFL10 or Laird Eccosorb 3mm × connector-height × 1mm thick Installed in physical gap between J4600 shell and antenna housing | Absorber tile | Physical install | 🟡 OPTIONAL | +8–12dB attenuation above 2.4GHz in the 6mm gap — additive to via fence. Zero BOM cost. Only practical if physical clearance allows tile placement. Install between MTB2 (antenna-facing shell tab) and antenna connector housing face. Particularly useful if via fence density is limited by routing constraints. |

Space-Constrained Implementation Tiers

Three valid cut-lines. Choose based on available PCB area. Each tier is testable. Lower tiers accept higher risk in exchange for fewer components.

| Tier | Parts | Space | FCC Risk | WiFi/BT Risk | Components Included |
| --- | --- | --- | --- | --- | --- |
| A Minimum | 7 items +2 free | ~8×0201 2×0402 1×0805 +net assigns | HIGH — PMIC harmonics visible on FCC scan above 100MHz. May need spot fixes at pre-compliance. | MEDIUM — CLK 24th harmonic filtered (-18dB). RefClk BT spur covered. PCIe TX 5GHz covered. DATA stubs unfiltered. | ✅ #1 CLK RC LPF: 22Ω + 22pF C0G on Pin 5 ✅ #2 VSS Pins 6,10,13,16 → GND_SD net only (free) ✅ #3 MTB shell tabs ×5 soldered → GND_SD (free) ✅ #4 R_GND_BRIDGE 0Ω + FB_GND_BRIDGE DNP footprints ✅ #5 RefClk CMC Pins 7+8: Murata DLW21SN471SQ2L ✅ #6 PCIe TX: TDK ACM2012-361 CMC + 100nF 0201 AC caps ×2 ✅ #7 1.8V dedicated LDO + 600Ω ferrite + 100nF + 100pF ❌ No DATA/CMD series R or 10pF shunt caps ❌ No 3.3V rail decoupling beyond LDO ❌ No PCIe RX filters, no optional items → Covers ~72% of total desense risk |
| B Recommended | ~22 items | +14×0201 +3×0402 vs Tier A | LOW — PMIC harmonics suppressed. Expected to pass FCC Part 15 without spot fixes. | LOW — All primary 2.4GHz + 5GHz sources mitigated. BT coexistence clean under normal conditions. | ✅ Everything in Tier A, plus: ✅ #8 10pF C0G shunt on DATA0-3 + CMD pull-ups ×5 ✅ #9 3.3V: 10µF X5R + 100nF X7R + 100pF C0G (skip ferrite if space tight) ✅ #10 22Ω series R on DATA0 + CMD (minimum — add DATA1-3 if space allows) ✅ #11 PCIe RX AC caps 0201 ×2 on Pins 14+15 ✅ #12 100Ω + 100nF on Pin 2 (PwrEn/FnRST PCIe mode) ❌ No RefClk VDD supply filter (3-terminal cap) ❌ No PCIe RX CMC ❌ 1.2V decoupling: DNP footprints only → Covers ~90% of total desense risk |
| C Full | ~34 items | +12×0201 +2×0402 vs Tier B | VERY LOW — Full decoupling on all rails. Expect clean FCC scan first pass. | VERY LOW — All sources mitigated. Lab validation expected to pass without iterations. | ✅ Everything in Tier B, plus: ✅ #10 22Ω series R on DATA1, DATA2, DATA3 (remaining 3 lines) ✅ #9 3.3V ferrite: Murata BLM15PX601SN1D (add to Tier B decoupling) ✅ #13 RefClk VDD: 1800Ω FB + 470nF 3-terminal cap (DNP→pop) ✅ #14 PCIe RX CMC: TDK ACM2012-361-2P ✅ #15 1.2V decoupling: populate if PCIe Express confirmed enabled ✅ #16 SW/DE: 10k pull-up + 100pF C0G shunt ×2 ✅ #17 CLKREQ# 4.7k pull-up swap + reuse 10pF shunt ✅ #18 RF absorber tile in 6mm gap if physical clearance allows → Covers >98% of total desense risk |

What to Cut First Under Space Pressure

| NEVER cut these Removing any = guaranteed failure 🔴 CLK 22Ω + 22pF C0G — Pin 5 🔴 VSS → GND_SD net assignment 🔴 MTB shell tabs → GND_SD 🔴 GND bridge placeholder footprints 🔴 RefClk CMC (DLW21SN471) 🔴 PCIe TX CMC + 0201 AC caps 🔴 1.8V dedicated LDO | Cut here first if needed Run CST trace length sweep first to quantify 🟠 DATA2 + DATA3 22Ω series R Less active lines — cut last two 🟠 3.3V ferrite (keep 3 decoupling caps) Caps do most of the work 🟠 PCIe RX CMC TX CMC covers 80%+ of PCIe issue 🟠 PwrEn 100Ω+100nF Pin 2 Only if reset edge confirmed <1ns 🟠 RF absorber tile in gap Additive — not primary mitigation | Cut freely — low risk DNP footprint — populate only if lab shows issue 🟡 RefClk VDD 3-terminal cap Spur present but narrower without it 🟡 1.2V PCIe decoupling Only if PCIe Express is actually used 🟡 SW/DE 100pF shunts Slow mechanical signal — low risk 🟡 CLKREQ# 4.7k pull-up Only if L1.2 power state in use 🟡 RF absorber in 6mm gap Nice-to-have, not essential |
| --- | --- | --- |

CST Simulation Results → Priority Updates

Three targeted sims, ~15 min total. Results may promote SHOULD components to MUST or demote to OPTIONAL based on your actual trace geometry.

| Sim | Netlist Block | Time | How Result Updates Your Priority Tier |
| --- | --- | --- | --- |
| CLK filter C0G vs X7R | CLK_FILTER CLK_DIELECTRIC sweep | ~2 min | At 2496MHz: C0G should show -18dB, X7R -12dB. If VDDIO=3.3V, X7R degrades further to -8dB — 10dB gap vs C0G. This confirms C0G is non-negotiable and gives exact margin vs FCC Part 15 limit. If your margin is <3dB with C0G: increase shunt cap to 33pF. |
| DATA stub trace length sweep | DATA0_STUB .STEP PARAM TRACE_LEN LIST 5 10 15 20 25 | ~3 min 5 runs | Stub resonance shifts with trace length. If resonance falls BELOW 2GHz at your trace length: promote rows 8+10 (shunts + series R) from SHOULD to MUST. If resonance above 3GHz: keep as OPTIONAL. This single sim determines your tier A vs tier B cut line for the DATA/CMD filters. |
| RefClk CMC SCC21 | REFCLK_CMC mixed-mode SCC21 at 2400MHz | ~2 min | SCC21 at 2400MHz: should show <-20dB with DLW21SN471SQ2L. If only -15dB (marginal): add 1800Ω FB_REFCLK_VDD supply filter — tightens the harmonic, improving effective SCC21 by 3-5dB. This determines whether row 13 (RefClk VDD filter) moves from OPTIONAL to SHOULD for your specific design. |

The Single Most Important Rule If you can only add one thing beyond the free net assignments and soldering: → Populate the CLK RC LPF: 22Ω + 22pF C0G on Pin 5. Two 0201 components. It eliminates your single largest desense source. Without it, the 24th CLK harmonic at 2496MHz reaches your WiFi and BT antenna at near-full amplitude on every SD transfer. No other component on this list provides the same step-change improvement. Second rule: the GND domain assignments (VSS→GND_SD, MTB→GND_SD) are free. Check your PCB copper pours now, before fabrication. A net assignment error requires a board spin to fix.