J4600  —  Amphenol 10102166A812A

microSD Express  |  EMI & Desense Mitigation Summary

System: WiFi + BT Only  |  SD CLK 104MHz  |  6mm Antenna Proximity  |  Rev 3.0

| WiFi + BT Only No cellular, no GPS Desense concern = 2.4GHz only | SD CLK 104MHz 24th harmonic = 2496MHz Dead center WiFi ch1 + BT band | PCIe RefClk 100MHz 24th harmonic = 2400MHz BT lower band edge | Primary Constraint FCC Part 15 emissions + 2.4GHz self-interference |
| --- | --- | --- | --- |

| SDC Only Filter: CLK harmonics | SDC / PCIe Dual-mode — filter differs per mode | PCIe Only Filter: 2.5/5GHz differential | Power 3-stage decoupling + ferrite | GND Domain: GND_SD not GND_ANT | Mech RC debounce only |
| --- | --- | --- | --- | --- | --- |

| Pin | Signal | Mode | Add to Schematic | Value | Package | EMI / Desense Rationale |
| --- | --- | --- | --- | --- | --- | --- |
| PIN 5 — CLK ¦ SDC MODE ONLY ¦ 22Ω + 22pF C0G (updated from 208MHz design) |  |  |  |  |  |  |
| 5 | CLK | SDC | Series RC LPF at SoC output 22Ω + 22pF C0G shunt to GND | 22Ω series 22pF to GND | 0201 / 50V 0201 C0G 50V | 104MHz CLK — 24th harmonic lands at 2496MHz, dead center in WiFi ch1 (2412MHz) and BT band (2402-2480MHz). This is the primary self-interference source in your system. RC LPF fc=330MHz: passes 104MHz at -0.3dB, attenuates 2496MHz by -18dB. NOTE: Changed from 33Ω+10pF used in 208MHz designs — lower series R preserves signal integrity at 104MHz while larger shunt cap gives better HF rejection. C0G mandatory: X7R loses 50% capacitance under 1.8V VDDIO bias, shifting fc to ~470MHz and leaving the 24th harmonic almost unfiltered. SDC mode only — benign in PCIe Express mode. |
| PIN 3 — CMD / Response ¦ SDC Only |  |  |  |  |  |  |
| 3 | CMD / CR | SDC | 22Ω series + 47k pull-up to VDDIO + 10pF C0G shunt across pull-up | 22Ω series 47k to VDDIO 10pF to GND | 0201 / 50V 0402 / 10V 0201 C0G 10V | Open-drain bidirectional. 22Ω slews edge, cutting harmonics above 500MHz by >10dB. 47k pull-up holds CMD at VDDIO when SoC tristates — floating CMD near 6mm antenna resonates at GHz. 10pF C0G shunt across the 47k is the most commonly missed component: kills stub RF susceptibility above 300MHz, prevents the 47k-terminated trace from receiving 2.4GHz energy and injecting into the SoC. C0G mandatory. |
| PINS 7 + 8 — DATA0/DATA1 (SDC) ¦ PCIe REFERENCE CLOCK +/- (PCIe Express) ← DUAL MODE |  |  |  |  |  |  |
| 7 | DATA0 (SDC) RefClk+ (PCIe) | SDC/PCIe | SDC: 22Ω + 47k pull-up + 10pF C0G PCIe: CMC on RefClk pair + FB + 3-term cap on RefClk VDD | SDC: 22Ω + 47k + 10pF PCIe RefClk: CMC 470Ω@100MHz FB 1800Ω@100MHz 3-term cap 470nF | SDC: 0201+0402+C0G CMC: 0805 100Ω diff FB: 0402 100mA Cap: 0402 X7R 10V | DUAL MODE — most filter-critical pin on J4600. SDC: standard DATA0 network — 22Ω slew + 10pF C0G shunt kills 2.4GHz stub susceptibility. PCIe EXPRESS: this pin becomes RefClk+. 100MHz RefClk 24th harmonic at 2400MHz is 2MHz below BT lower band edge — BT frequency hopping hits this spur on every sweep through lower channels causing BT throughput degradation. CMC (Murata DLW21SN471SQ2L) provides >20dB CM rejection while adding <0.3dB differential insertion loss — purely common-mode, transparent to the RefClk differential signal. 1800Ω FB + 470nF 3-terminal cap on RefClk VDD supply: >60dB supply rejection — prevents PMIC noise adding phase noise to RefClk, which would spread the harmonic and widen the interference window into BT channels. Do NOT use a series R in PCIe RefClk mode — it degrades phase noise. |
| 8 | DATA1 (SDC) RefClk- (PCIe) | SDC/PCIe | SDC: 22Ω + 47k pull-up + 10pF C0G PCIe: shares CMC with Pin 7 (symmetric placement mandatory) | SDC: 22Ω + 47k + 10pF PCIe: shares CMC_REFCLK with Pin 7 | SDC: 0201+0402+C0G Shared CMC — 0805 | DUAL MODE — mirror of Pin 7. PCIe EXPRESS: RefClk- shares the 2-line CMC. Must mirror Pin 7 component placement within 5 mils — any asymmetry converts differential RefClk to common-mode, completely negating the CMC and spreading harmonic energy into both 2.4GHz WiFi and BT simultaneously. |
| PIN 1 — DATA2 (SDC) ¦ PCIe CLKREQ# / L1 PM Sub-states (PCIe) |  |  |  |  |  |  |
| 1 | DATA2 (SDC) CLKREQ# (PCIe) | SDC/PCIe | SDC: 22Ω + 47k pull-up + 10pF C0G PCIe: 4.7k pull-up + 10pF C0G (no series R in PCIe mode — open-drain) | SDC: 22Ω+47k+10pF PCIe: 4.7k+10pF | 0201/50V + 0402 + 0201 C0G 10V | DUAL MODE — filter topology changes per mode. SDC: standard DATA2 filter. PCIe: CLKREQ# is open-drain active-low. The 22Ω series R from SDC mode must NOT be in the CLKREQ# path — it slows the assertion edge, directly impacting PCIe L1.2 power state exit latency. Use 4.7k pull-up per PCIe spec. The 10pF C0G shunt is mandatory in both modes: without it the 4.7k-terminated CLKREQ# stub near the 6mm antenna receives 2.4GHz energy and causes spurious L1 exit events, manifesting as intermittent link failures during WiFi/BT activity nearby. |
| PIN 2 — DATA3 / CD (SDC) ¦ Power Enable / Functional Reset (PCIe) |  |  |  |  |  |  |
| 2 | DATA3/CD (SDC) PwrEn/FnRST (PCIe) | SDC/PCIe | SDC: 22Ω + 47k pull-up + 10pF C0G PCIe: 100Ω + 100nF slew control (larger cap — power switching transient) | SDC: 22Ω+47k+10pF PCIe: 100Ω+100nF | 0201/50V + 0402 + C0G / X7R 10V | DUAL MODE — filter values differ significantly. SDC: standard DATA3 filter, also debounces mechanical card-detect bounce (broadband burst to 100MHz without the 10pF shunt). PCIe: Power Enable / Functional Reset — fast reset edge <1ns generates broadband harmonics 1MHz-1GHz on this trace 6mm from antenna. 100Ω+100nF (fc=16kHz) slows edge to ~10µs, eliminating all harmonic content while remaining transparent to PCIe reset timing (spec allows up to 100ms). 100nF shunt (not 10pF) needed here — higher current power-enable transient requires larger damping cap. |
| POWER RAILS — 3.3V (Pin 4), 1.8V VDDIO (Pin 9), 1.2V PCIe (Pin 17) |  |  |  |  |  |  |
| 4 | 3.3V Supply | Power | Ferrite on rail + 3-stage decoupling | 600Ω@100MHz ferrite 10µF bulk 100nF mid 100pF HF (C0G) | 0402 500mA 0402 X5R 10V 0201 X7R 10V 0201 C0G 10V | PMIC switching harmonics (1-25MHz) ride the 3.3V rail to the connector and radiate 6mm from antenna. Ferrite blocks PMIC harmonics before they reach the connector. Three decoupling stages mandatory: 10µF handles card-insertion surge current (prevents 50-200MHz droop oscillation); 100nF kills CLK-synchronous supply ripple at 100-300MHz; 100pF C0G kills sub-ns spikes above 300MHz. All three show up as spurs on FCC Part 15 scan without mitigation. C0G mandatory for 100pF. |
| 9 | 1.8V VDDIO | Power | Dedicated switchable LDO + ferrite + 2-stage decoupling | Switchable LDO 1.8V/3.3V 600Ω@100MHz ferrite 100nF mid 100pF HF (C0G) | SOT23/LGA 0402 300mA 0201 X7R 10V 0201 C0G 10V | VDDIO powers DATA/CLK/CMD I/O buffers. MUST be a dedicated switchable LDO — sharing with any other domain means a 3.3V-to-1.8V mode switch glitches all signal levels simultaneously. Ferrite breaks CLK feedback loop: CLK switching → VDDIO ripple → CLK jitter → harmonic spreading into 2.4GHz WiFi/BT band. Without it the 2496MHz spur widens in frequency and is harder to filter. |
| 17 | 1.2V PCIe | Power | Ferrite + 2-stage decoupling (DNP if PCIe mode not used) | 600Ω@100MHz ferrite 100nF mid 100pF HF (C0G) | 0402 500mA 0201 X7R 10V 0201 C0G 10V | PCIe Express SerDes VDDIO. PHY switching at 2.5GHz back-propagates through this rail. Your schematic shows Pin 17 with X (no-connect) — if PCIe Express is disabled for this slot, 1.2V may be unused. Still place DNP footprints for all three components — costs nothing, avoids a board spin if Express mode is enabled later. |
| GROUND PINS — VSS_1 to VSS_4 ¦ Pins 6, 10, 13, 16 |  |  |  |  |  |  |
| 6 10 13 16 | VSS Ground (×4 pins) | GND | ALL VSS → GND_SD only NOT GND_ANT Test point on Pin 6 | 0Ω to GND_SD Via stitch ≤1.5mm | 0402 test point 4× vias per pad | All VSS pins carry SD and PCIe return current. MUST connect to GND_SD (digital ground domain), NEVER GND_ANT. If any VSS touches GND_ANT copper, SD CLK harmonics and PCIe RefClk harmonics flow through the antenna return path and couple directly into your WiFi/BT antenna feed — this is the conducted interference path and is the hardest failure mode to debug because it shows no obvious radiated source. Add test point on one VSS pin for current-probe measurement during lab characterization. |
| CONNECTOR SHELL — MTB Pads ¦ Right-side shield tabs |  |  |  |  |  |  |
| MTB 1-5 | Shell GND | GND | Solder ALL MTB pads → GND_SD 1.0mm via pitch: antenna-facing tab 1.2mm via pitch: remaining tabs | Solder all tabs Via fence GND_SD RF absorber optional | PCB vias all layers 1.0mm pitch (antenna side) 1.2mm pitch (others) | Metal connector shell is the largest broadband radiating surface in the assembly — directly relevant to FCC Part 15 emissions. Unsoldered tab leaves a gap >6mm acting as a slot antenna at 2.4GHz, which in your system means direct radiation into your WiFi/BT band. All tabs to GND_SD (not GND_ANT). Optional: TDK IFL10 or Laird Eccosorb absorber tile (3mm × 1mm) placed in the 6mm physical gap between connector shell and antenna connector — adds 8-12dB attenuation above 2.4GHz beyond what via fence alone provides. |
| GND DOMAIN ISOLATION — GND_SD ↔ GND_ANT Bridge (most critical placeholder) |  |  |  |  |  |  |
| GND Bridge | GND_SD → GND_ANT | GND | 0Ω bridge (populated) in series with DNP ferrite — separate footprints Between GND_SD and GND_ANT | 0Ω (populated) 600Ω@100MHz 1A (DNP default) | 0201 / cuttable 0603 / 1A rated | Default: R_GND_BRIDGE = 0Ω populated, FB_GND_BRIDGE = DNP. Lab debug: cut R_GND_BRIDGE pad, populate ferrite. Ferrite = 0Ω DC (WiFi/BT antenna DC ground remains valid) + >20dB RF isolation at 100MHz+, blocking SD/PCIe switching transients from reaching the antenna return path. Zero PCB rework needed — this is the single most useful debug tool in the design. MUST be two separate footprints in series: a single 0Ω pad cannot be swapped for a ferrite without a board spin. |
| SW, DE — Mechanical Card Detect (NOT mode-switching signals) |  |  |  |  |  |  |
| SW DE | Card Detect (Mechanical) | Mech | 10k pull-up to VDDIO + 100pF C0G debounce shunt | 10k to VDDIO 100pF to GND | 0402 / 10V 0201 C0G 10V | SW and DE are mechanical insertion-detection switches only — NOT mode-switching signals (corrected from Rev 1.0). Simple RC debounce: 10k pull-up holds the line defined when no card is present; 100pF C0G shunt kills RF susceptibility of the pull-up stub (same principle as DAT pull-up shunts). No series R needed — these are slow mechanical signals with no edge-rate concern. |
| PCIe DIFFERENTIAL PAIRS — TX_P/TX_N (Pins 11,12) ¦ RX_P/RX_N (Pins 14,15) |  |  |  |  |  |  |
| 11 12 | PCIe TX (+/-) | PCIe | 100nF AC coupling cap (×2) + CMC on TX pair Pin 11: C_TX_P ¦ Pin 12: C_TX_N | 100nF AC couple (×2) 360Ω@100MHz CMC | 0201 X7R 16V (×2) 0805 100Ω diff | PCIe Gen1 at 2.5GHz or Gen2 at 5GHz. At 6mm from WiFi/BT antenna, the TX common-mode radiates directly into 2.4GHz WiFi (Gen1) or 5GHz WiFi (Gen2). AC coupling caps mandatory per PCIe spec — block DC offset from driving common-mode current. CMC provides >20dB CM rejection while adding <0.5dB differential IL. CRITICAL: 0201 mandatory for AC caps — 0402 resonates at 3-4GHz, creating an emission peak directly in WiFi 5GHz band rather than suppressing. Place CMC within 3mm of connector pins. Mirror TX_P and TX_N cap placement within 5 mils — asymmetry converts differential to CM, negating the CMC. |
| 14 15 | PCIe RX (+/-) | PCIe | 100nF AC coupling cap (×2) + CMC on RX pair | 100nF AC couple (×2) 360Ω@100MHz CMC | 0201 X7R 16V (×2) 0805 100Ω diff | RX pair picks up SD CLK 24th harmonic at 2496MHz as common-mode ingress — this is the same spur that desenses your WiFi. CMC on RX prevents this from entering the PCIe RX as a noise floor elevation and also prevents 2.4GHz WiFi energy from coupling back into the RX pair via common-mode. |

104MHz SD CLK — Harmonic Map for WiFi + BT System

With no cellular or GPS in the system, only harmonics that land in the WiFi (2.4GHz / 5GHz) and Bluetooth (2.402–2.480GHz) bands are operationally relevant. FCC Part 15 emissions remain a constraint for all harmonics regardless.

| Harmonic | Frequency | Band Impact | Concern | Mitigation |
| --- | --- | --- | --- | --- |
| 2nd–23rd | 208–2392 MHz | No WiFi/BT bands impacted | FCC emissions only — no receiver in system affected | Standard power rail ferrites + CLK RC LPF handles this range. No specific action needed for WiFi/BT. |
| 23rd | 2392 MHz | 8MHz below BT lower edge (2402MHz) | LOW RISK — BT lower band adjacent | RC LPF on CLK (fc=330MHz) provides -17dB at 2392MHz. Acceptable. |
| 24th ⚠ | 2496 MHz | WiFi ch1 (2412MHz) + BT band center | CRITICAL — primary WiFi/BT self-interference source | 22Ω + 22pF C0G LPF (fc=330MHz) → -18dB at 2496MHz. GND_SD/GND_ANT isolation. Via fence on MTB shell tabs. |
| 25th | 2600 MHz | WiFi ch11–13 upper edge | MEDIUM — partially inside 2.4GHz band | Same CLK LPF provides -19dB. Covered. |
| 48th | 4992 MHz | 5GHz WiFi (4915–5825MHz) | MEDIUM — inside 5GHz WiFi band | PCIe TX CMC (TDK ACM2012-361) provides >20dB CM rejection at 5GHz. AC coupling caps on TX/RX pairs. |

100MHz PCIe RefClk — Harmonic Map for WiFi + BT System

RefClk runs continuously while a PCIe Express card is inserted — sustained always-on interference source unlike SD CLK which only runs during transfers.

| Harmonic | Frequency | Band Impact | Concern | Mitigation |
| --- | --- | --- | --- | --- |
| 2nd–23rd | 200–2300 MHz | No WiFi/BT bands impacted | FCC emissions scan only | Power rail ferrites + FB_REFCLK_VDD 1800Ω covers this range. |
| 24th ⚠ | 2400 MHz | BT lower band edge (2402MHz) 2MHz separation | HIGH — BT freq hopping sweeps directly across this spur every cycle | CMC on RefClk pair (Murata DLW21SN471SQ2L) + 1800Ω FB + 470nF 3-terminal cap on RefClk VDD supply. >60dB total supply rejection. |
| 25th | 2500 MHz | WiFi ch2 (2417MHz) adjacent | MEDIUM — close to WiFi 2.4GHz lower channels | CMC on RefClk pair covers. -20dB CM rejection at 2.5GHz. |
| 50th | 5000 MHz | 5GHz WiFi band | LOW — 50th harmonic is typically low amplitude | PCIe TX CMC covers 5GHz range. Unlikely to be a compliance issue. |

Components Removed vs Previous LTE/GPS Designs — WiFi+BT System

REMOVED FROM BOM — Not Needed for WiFi + BT Only System FB_GPS_VDD (1800Ω GPS LNA supply ferrite) Reason: No GPS receiver in system. Was protecting GPS L1 at 1575.42MHz — irrelevant without GPS LNA. LTE band harmonic analysis and LTE-specific filter constraints Reason: No cellular receiver. However NOTE: the same CLK harmonics that would hit LTE bands still appear as out-of-band spurs on FCC Part 15 emissions scans — filters still needed for certification. 1800Ω FB_PCIE_REF (second ferrite on PCIe RefClk supply for GPS L1 protection) Reason: Was specifically for GPS L1 at 1575MHz (16th RefClk harmonic). Not needed. Keep the standard 1800Ω FB_REFCLK_VDD — that is for general supply noise and BT protection.

The 3 Components You Cannot Skip — WiFi + BT System

| ① CLK Pin 5 22Ω + 22pF C0G at SoC output 24th harmonic at 2496MHz is dead center in your WiFi ch1 and BT band. This single component is your primary 2.4GHz self-interference fix. Place within 1mm of SoC CLK output pin. C0G mandatory. | ② CMC on RefClk Pins 7+8 Murata DLW21SN471SQ2L + 1800Ω FB 100MHz RefClk 24th harmonic at 2400MHz is 2MHz below BT lower band edge — BT frequency hopping hits this spur constantly. Sustained always-on interference while card is inserted, not just during transfers. | ③ GND Bridge Placeholder R_GND_BRIDGE (0Ω) + FB_GND_BRIDGE (DNP) Two separate footprints in series between GND_SD and GND_ANT. Default 0Ω. Lab fix: cut and populate 600Ω 1A ferrite — zero rework. The single most valuable debug tool if conducted interference reaches your WiFi/BT antenna port. |
| --- | --- | --- |

Quick Reference BOM — All Components for J4600 (WiFi + BT System)

GPS LNA ferrite removed vs LTE/GPS designs. All other components unchanged.

| Ref Des | Pin(s) | Part Number | Value | Package | Function |
| --- | --- | --- | --- | --- | --- |
| R_CLK | 5 | Vishay CRCW020122R0FK | 22Ω | 0201 | CLK series R — LPF with C_CLK. fc=330MHz. Updated for 104MHz CLK (was 33Ω in 208MHz designs). |
| C_CLK | 5 | Murata GRM0335C1H220J | 22pF C0G | 0201 50V | CLK shunt cap — LPF. C0G mandatory. Updated to 22pF (was 10pF in 208MHz designs) for better 2.4GHz rejection. |
| R_CR | 3 | Vishay CRCW020122R0FK | 22Ω | 0201 | CMD series slew resistor. |
| RPU_CR | 3 | Vishay CRCW040247K0FK | 47k | 0402 10V | CMD open-drain pull-up to VDDIO. |
| C_CR | 3 | Murata GRM0335C1H100J | 10pF C0G | 0201 10V | CMD pull-up shunt — kills 2.4GHz stub susceptibility. |
| R_D0–R_D3 | 7,8,1,2 | Vishay CRCW020122R0FK | 22Ω (×4) | 0201 50V | DATA0-3 series slew resistors. Independent for each line — never shared. |
| RPU_D0–D3 | 7,8,1,2 | Vishay CRCW040247K0FK | 47k (×4) | 0402 10V | DATA0-3 pull-ups to VDDIO. SD spec mandatory. |
| C_D0–D3 | 7,8,1,2 | Murata GRM0335C1H100J | 10pF C0G (×4) | 0201 10V | DATA pull-up shunt caps — kills 2.4GHz stub susceptibility on each line. C0G mandatory. |
| CMC_REFCLK | 7+8 | Murata DLW21SN471SQ2L | 470Ω@100MHz | 0805 100Ω diff | PCIe RefClk pair CMC. Suppresses 24th harmonic (2400MHz) from reaching BT band. Transparent to RefClk differential signal. |
| FB_REFCLK_VDD | RefClk VDD | Murata BLM15PX182SN1D | 1800Ω@100MHz | 0402 100mA | RefClk VDD supply ferrite. Prevents PMIC noise from adding phase noise to RefClk — widens harmonic and hits more BT channels. |
| C_REFCLK_VDD | RefClk VDD | Murata NFM18CC474R1H3D | 470nF 3-term | 0402 X7R 10V | 3-terminal cap on RefClk VDD. Pi-filter topology — no resonance. >60dB supply rejection combined with FB. |
| C_TX_P,TX_N C_RX_P,RX_N | 11,12 14,15 | Murata GRM033R71C104K | 100nF (×4) | 0201 X7R 16V | PCIe differential pair AC coupling caps. Mandatory per PCIe spec. 0201 mandatory — 0402 resonates at 3-4GHz. |
| CMC_TX CMC_RX | 11+12 14+15 | TDK ACM2012-361-2P-T002 | 360Ω@100MHz (×2) | 0805 100Ω diff | PCIe TX and RX pair CMCs. >20dB CM rejection at 2.5GHz — suppresses WiFi 2.4GHz/5GHz interference. |
| FB_3V3 FB_1V8 FB_1V2 | 4, 9, 17 | Murata BLM15PX601SN1D | 600Ω@100MHz (×3) | 0402 500mA | Power rail ferrites on 3.3V, 1.8V, 1.2V. Block PMIC switching harmonics. Appear as FCC emission spurs without mitigation. |
| C_xV_BULK | 4 (3.3V) | Murata GRM188R60J106M | 10µF X5R | 0402 10V | 3.3V bulk decoupling. Handles card-insertion surge. Prevents 50-200MHz droop oscillation. |
| C_xV_MID (×3) | 4,9,17 | Murata GRM033R71A104K | 100nF X7R (×3) | 0201 10V | Mid-freq decoupling on all three rails. 0201 mandatory — 0402 SRF=80MHz, misses 100-300MHz target. |
| C_xV_HF (×3) | 4,9,17 | Murata GRM0335C1H101J | 100pF C0G (×3) | 0201 10V | HF decoupling. C0G mandatory on all three — X7R degrades under bias and fails above 300MHz. |
| R_GND_BRIDGE | GND bridge | Vishay CRCW02010000Z0ED | 0Ω | 0201 | GND domain bridge — populated by default. Cut and replace with FB_GND_BRIDGE if conducted interference found in lab. |
| FB_GND_BRIDGE | GND bridge | Murata BLM18PG601SN1D | 600Ω@100MHz 1A (DNP) | 0603 1A | GND bridge ferrite — DNP by default. Provides >20dB RF isolation GND_SD↔GND_ANT while maintaining 0Ω DC. |
| RPU_SW RPU_DE | SW, DE | Vishay CRCW040210K0FK | 10k (×2) | 0402 10V | Card detect pull-ups. Mechanical switch — no edge rate concern. Holds line defined when no card present. |
| C_SW C_DE | SW, DE | Murata GRM0335C1H101J | 100pF C0G (×2) | 0201 10V | Card detect debounce shunt. Kills RF susceptibility of mechanical switch stub near antenna. |