J4600 Filter Priority Simulation

CST Design Studio — Combined Schematic-Level Netlist

No layout required  |  No 3D solver  |  All networks in one sweep  |  ~15 min setup

WHAT THIS SIMULATION DOES Runs all J4600 filter networks simultaneously from a normalized aggressor source. Plots S21 (isolation) for each network at 2496MHz (24th CLK harmonic) and 2400MHz (24th RefClk harmonic). Higher S21 at the marker frequency = worse isolation = higher implementation priority. Also runs C0G vs X7R degradation comparison and 0201 vs 0402 AC cap resonance comparison. Pure AC circuit simulation — no EM, no layout, no mesh. Runs in seconds.

Setup in CST Design Studio

| 1 | Open CST and create a new Design Studio project Launch CST Studio Suite File → New → Project Select: Design Studio (Schematic & Circuit Simulation) Do NOT select MWS (Microwave Studio) — that is the 3D solver, not needed here. |
| --- | --- |

| 2 | Import the netlist file File → Import → CST Schematic Netlist (.cst) Navigate to: J4600_CST_Combined.cst Click Open — CST will parse all 7 blocks and place them as schematic sheets. You will see 7 tabs: CLK_FILTER, DATA0_STUB, CMD_FILTER, REFCLK_CMC, PCIE_TX, PWR_3V3, COMBINED_PRIORITY |
| --- | --- |

| 3 | Set the frequency sweep Simulation → Solver Settings → Frequency Domain Fstart = 10 MHz ¦ Fstop = 6 GHz ¦ Points = 3000 Solver: SPICE AC (not FEM, not TDR) This sweep covers: CLK fundamental (104MHz), all harmonics to 6GHz, WiFi 2.4GHz and 5GHz bands. |
| --- | --- |

| 4 | Configure parameter sweeps Simulation → Parameter Sweep → Add Sweep Sweep 1 (CLK_DIELECTRIC): RUN_COG=1/RUN_X7R=0 then RUN_COG=0/RUN_X7R=1 → This compares 22pF C0G (correct) vs 11pF effective X7R (degraded under 1.8V bias) Sweep 2 (AC_CAP_PACKAGE): RUN_0201=1/RUN_0402=0 then RUN_0201=0/RUN_0402=1 → This shows 0201 flat response vs 0402 resonance peak at 3-4GHz Optional Sweep 3 (TRACE_LENGTH): uncomment the sweep block for sensitivity analysis |
| --- | --- |

| 5 | Run the simulation Simulation → Start Simulation (or press F8) Expected runtime: 5-15 seconds for all sweeps combined (pure SPICE AC) No mesh generation, no convergence issues — circuit simulation is deterministic. |
| --- | --- |

| 6 | Load the post-processing plots Post Processing → Load Post Processing List → select the [POSTPROCESSING] blocks from the .cst file OR manually: Results → S-Parameters → select curves as listed below Six pre-defined plots will appear (see plots section below) |
| --- | --- |

What to Look For in Each Plot

| Plot Name | Curves | Key Markers | How to Read It |
| --- | --- | --- | --- |
| PRIORITY_2496MHz | CLK S21 DATA0 S21 CMD S21 PWR S21 | 2.496 GHz | All four curves on one graph. At the 2496MHz marker, read S21 for each. The curve with the HIGHEST (least negative) S21 = worst filter = highest priority. Target: all curves below -15dB at this marker. |
| PRIORITY_2400MHz | RefClk SCC21 TX SCC21 | 2.4 GHz | Common-mode rejection at BT band edge. Both curves should be below -20dB at 2.4GHz marker. If either is above -20dB the CMC is insufficient — try DLW21SN471SQ2L vs a lesser CMC to see the difference. |
| CLK_COG_VS_X7R | C0G run X7R run | 104 MHz 330 MHz 2.496 GHz | Two overlaid S21 curves. At 104MHz both should be near 0dB (filter passes CLK). At 2496MHz: C0G curve should be -18dB, X7R curve -12dB. The 6dB gap is the difference between passing and failing FCC emissions. |
| AC_CAP_RESONANCE | 0201 SDD21 0402 SDD21 | 2.5 GHz 5.0 GHz | 0201 curve should be flat to -1dB at 5GHz. 0402 curve will show a resonance PEAK (S21 rises above 0dB) around 3-4GHz — this is the emission spike that appears on your FCC scan in the 5GHz WiFi band. |
| STUB_RESONANCE | With 10pF S11 Without 10pF S11 | 2.4 GHz 2.496 GHz | S11 = how much the stub reflects energy (high S11 = good, stub doesn't absorb). Without 10pF: expect a DIP in S11 at 2-3GHz (stub resonates = absorbs WiFi energy = injects into SoC). With 10pF: S11 stays high = stub terminated = no absorption. |
| REFCLK_MIXEDMODE | SDD21 SCC21 SDC21 | 100 MHz 2.4 GHz | Three curves: SDD21 at 100MHz must be >-0.5dB (CMC transparent to RefClk). SCC21 at 2.4GHz must be <-20dB (CMC rejecting BT harmonic). SDC21 at all freqs should be <-30dB (mode conversion = layout quality indicator). |
| PWR_IMPEDANCE | Full 3-stage Z No HF cap No mid cap | 10 MHz 29 MHz 300 MHz 920 MHz | Impedance Z(ohm) vs frequency. Full 3-stage should stay below 1 ohm from 10MHz to 3GHz. Remove HF cap: Z rises above 1 ohm above 300MHz. Remove mid cap: Z rises above 1 ohm above 30MHz. Shows which cap stage covers which decade. |

Expected Priority Ranking — Read from PRIORITY_2496MHz Plot

After running the simulation, read S21 at the 2496MHz marker for each curve. The ranking below is what the simulation should confirm:

| Rank | Network | Expected S21 @ 2496MHz | Expected S21 @ 2400MHz | Tier | Action if Skipped |
| --- | --- | --- | --- | --- | --- |
| #1 | CLK LPF — Pin 5 22Ω + 22pF C0G | −18 dB | −17 dB | 🔴 MUST | Certain FCC fail. WiFi ch1 + BT desense on every SD transfer. |
| #2 | RefClk CMC — Pins 7+8 Murata DLW21SN471 | — (CM only) | −22 dB | 🔴 MUST | Always-on BT interference while PCIe card inserted. BT hops across 2400MHz spur constantly. |
| #3 | PCIe TX CMC — Pins 11+12 TDK ACM2012-361 | −21 dB | −20 dB | 🔴 MUST | PCIe 2.5GHz fundamental inside 5GHz WiFi. FCC emissions peak. |
| #4 | DATA0 pull-up stub Pin 7 — 10pF C0G shunt | −20 dB (w/ shunt cap) | −19 dB | 🟠 SHOULD | Without shunt: S11 dip at 2-3GHz. Stub receives WiFi energy, injects into SoC. |
| #5 | 3.3V power rail Ferrite + 3-stage decoupling | −35 dB | −34 dB | 🟠 SHOULD | PMIC harmonics on FCC scan. Bulk decoupling prevents insertion surge oscillation. |
| #6 | CMD stub — Pin 3 22Ω + 10pF C0G shunt | −20 dB | −19 dB | 🟡 OPTIONAL | Lower CMD activity vs CLK. Risk proportional to trace length. Keep 10pF shunt, skip 22Ω if space tight. |

Key Parameters to Adjust for Your Design

TRACE_LEN — Most Impactful Parameter to Sweep The stub resonance frequency (visible in STUB_RESONANCE plot) shifts directly with trace length. Short trace (5mm): stub resonates at ~4-5GHz — above WiFi band, lower risk Medium trace (15mm): stub resonates at ~2-3GHz — inside WiFi/BT band, high risk Long trace (25mm): stub resonates at ~1.5-2GHz — below WiFi but still BT risk To run the trace length sweep: in the netlist, uncomment the SWEEP TRACE_LENGTH block. Run with TRACE_LEN = 5, 15, 25 — the plot shows you exactly where your stub resonance lands. This result tells you whether the 10pF shunts on DATA/CMD are optional or essential for YOUR board.

VDDIO — Only Change if Your VDDIO is 3.3V Instead of 1.8V The X7R degradation model uses 1.8V as the bias voltage (C0G-X7R comparison). If your VDDIO is 3.3V: X7R degrades even MORE — change C_CLK_X7R from 11pF to ~8pF in the parameter block. At 3.3V bias, X7R 22pF → ~8pF effective. fc shifts from 330MHz to ~600MHz. The 24th harmonic at 2496MHz would then receive only -8dB attenuation vs -18dB with C0G.

What This Simulation Cannot Tell You

| Limitation | What You Need Instead |
| --- | --- |
| Cross-coupling between SD trace and antenna at 6mm gap | CST MWS 3D simulation — model connector footprint + 5mm trace + antenna stub. Quantifies coupling coefficient S21 between SD net and antenna port. Useful if you need to justify the 6mm spacing decision numerically. |
| CMC self-resonance and saturation behavior | Import Murata/TDK S4P Touchstone files into Design Studio. Replace the simplified RL model in the netlist with the measured data. Gives accurate CMC behavior above 1GHz where the simple RL model diverges. |
| PCIe eye diagram with filter in place | Export S2P from the PCIe TX block → import into HyperLynx or CST Board Signal Integrity → run eye diagram. Confirms the CMC and AC caps don't close the PCIe eye at Gen1 or Gen2. |
| Actual FCC emissions level in dBµV/m | Full 3D radiated emissions sim requires a model of the entire PCB, connector, and chassis. S21 from this sim gives you RELATIVE risk ranking, not absolute dBµV/m levels. Use the sim for priority decisions, not for predicting pass/fail margin. |
| GND domain isolation effectiveness | GND_SD vs GND_ANT isolation requires the PCB copper pour model — schematic sim cannot capture this. The GND bridge ferrite Z vs frequency can be simulated in Design Studio but the via fence and pour geometry requires 3D. |

Simulation Checklist

| □ | Import J4600_CST_Combined.cst → confirm 7 schematic tabs appear |
| --- | --- |
| □ | Run frequency sweep 10MHz–6GHz with SPICE AC solver |
| □ | Run CLK_DIELECTRIC sweep (C0G vs X7R) — confirm 6dB gap at 2496MHz |
| □ | Run AC_CAP_PACKAGE sweep (0201 vs 0402) — confirm resonance peak in 0402 curve at 3-4GHz |
| □ | Open PRIORITY_2496MHz plot — read S21 at marker for all curves — confirm CLK worst |
| □ | Open PRIORITY_2400MHz plot — confirm both CMC SCC21 < -20dB at 2400MHz |
| □ | Open STUB_RESONANCE plot — confirm resonance dip disappears when 10pF shunt added |
| □ | Open REFCLK_MIXEDMODE — confirm SDD21 > -0.5dB at 100MHz, SCC21 < -20dB at 2400MHz |
| □ | If trace length known: run TRACE_LENGTH sweep — find your actual stub resonance frequency |
| □ | Export results table and compare to Expected Priority Ranking table above |